0xA000_0000

| address | W/R | items | value |
| --- | --- | --- | --- |
| 0x0000 | W/R | brightness | 0 - 100 |
| 0x0004 | W/R | NVG brightness | 0 - 100 default 10 |
| 0x0008 | W/R | NVG mode | 0 – normal mode 1 – NVG mode |
| 0x000C | W/R | auto_manu | 0 – auto select video 1 – manual select video |
| 0x0010 | W/R | osdCtrl | Byte 3 reserve 0 – no OSD 1 – overlay OSD Byte 2 right video 0 – no OSD 1 – overlay OSD Byte 1 center video 0 – no OSD 1 – overlay OSD Byte 0 left video 0 – no OSD 1 – overlay OSD |
| 0x0014 | W/R | video source | 0 – video source 0 1 - video source 1 |
| 0x0018 | W/R | contrast value | 0 – 100 convert to -255 ~ 255 (x- 50) x 255 /50 |
| 0x001C | W/R | color value | 0 – 100 convert to -255 ~ 255 (x- 50) x 255 /50 |
| 0x0100 | read only | status | Byte 3 reserve bit 3-0 -- sdi 3 inputs: 0 normal, 1 no signal bit 4 -- ddr read: 0 normal, 1 no read operation bit 5 -- ddr write: 0 normal, 1 no write operation bit 7-6 – display: 00 normal, 01 no signal, 10 error Byte 2 right video bit 3-0 -- sdi 3 inputs: 0 normal, 1 no signal bit 4 -- ddr read: 0 normal, 1 no read operation bit 5 -- ddr write: 0 normal, 1 no write operation bit 7-6 – display: 00 normal, 01 no signal, 10 error Byte 1 center video bit 3-0 -- sdi 2 inputs: 0 normal, 1 no signal bit 4 -- ddr read: 0 normal, 1 no read operation bit 5 -- ddr write: 0 normal, 1 no write operation bit 7-6 – display: 00 normal, 01 no signal, 10 error Byte 0 left video bit 3-0 -- sdi 1 inputs: 0 normal, 1 no signal bit 4 -- ddr read: 0 normal, 1 no read operation bit 5 -- ddr write: 0 normal, 1 no write operation bit 7-6 – display: 00 normal, 01 no signal, 10 error |

| OSD | address |  |  |
| --- | --- | --- | --- |
| Left | 0x7000_0000 |  |  |
| Centre | 0x7400_0000 |  |  |
| Right | 0x7800_0000 |  |  |
| reserve | 0x7C00_0000 |  |  |