# Communication Protocol - Serial Port # In-between B3VDU and B3VCU

# Change Index.

| Index | Date | Amendment contents | Version | Person |
| --- | --- | --- | --- | --- |
| 1 | 22 Feb 2022 | First Copy for release | 1.0.1 | Chen Yong |
| 2 | Jul 14, 2022 | NUC mode | 1.0.2 | Chen Yong |
| 3 | Aug 18, 2022 | OSD Variant | 1.0.3 | Chen Yong |

Table of Contents

1   Scope	3

2   Hardware Specification	3

3   System Architecture	4

3.1 Data package	4

4 Data Format	4

4.1 Key package (from VDU to HOST)	4

4.2 Response of “Key Package” (From Host to VDU)	5

4.3 SYSCTL (from HOST to VDU)	5

4.4 SYSINFO (from VDU to HOST)	6

4.5DEV Package	6

4.6 Response of “DEV Package”	6

# 1   Scope

This document stipulates communication protocol of serial port in-Between B3VDU and B3VCU. This document will be used as a guideline to serial port communication of B3 VDU and B3VCU according to the technical specification requirement.

# 2   Hardware Specification

Physical link:TIA/EIA-422

baudrate:115200 bits/s

data bits:8 bits

stop bits:1 bit

parity: no

flow control: no

mode: duplex

# 3   System Architecture

## 3.1 Data package

| header | Package ID | parameters | tail | Checksum | End symbol |
| --- | --- | --- | --- | --- | --- |
| $ | Ascii code | Ascii code. Insert ‘,’ between parameters | * | 2 HEX ascii | \r or \n or \0 or (any combination) |

checksum: the two HEX character as the checksum of the package. (the sum of all characters of package between ‘$’ and  ‘*’, exclude ‘$’ and ‘*’). It is not a necessary item. It can be empty.

Example:

$KEY,123,0A,01234567*10\0

$KEY,123,0A,01234567*\r\n\0

# 4 Data Format

## 4.1 Key package (from VDU to HOST)

| item | value | description |
| --- | --- | --- |
| $PKGID | $KEY |  |
| PAR1 | Integer: 0 to 255 | Package No |
| PAR2 | Hex: 8bits bit0: front bit1: rear bit2: IR bit3: NUC bit7: NUC mode (note: front and rear cannot be 1 at the same time in the package.) | Key (bit0-bit3) 0=released 1=pressed NUC mode: 0=auto 1=development |
| PAR3 | Hex: 32bits Bit0: memory bit1: logdata bit2: snsor bit3: current bit4: temperature bit5: pwm0 bit6: pwm1 bit7: keypad bit8: video out bit9: video in0 bit10: video in1 bit11: osd bit12: serial bit13: host | System error bits. 0: PASS 1: Fail |
| PAR4 | Integer | System Power On timer(unit:s) |
| *CHK | checksum |  |

This package will be send out per 100ms by the VDU. The VDU will set the “host” error bit if the response package is not received within 100ms.

## 4.2 Response of “Key Package” (From Host to VDU)

| item | value | description |
| --- | --- | --- |
| $PKGID | $KEYA | Acknowledge of package ‘KEY’ |
| PAR1 | Integer: 0 to 255 | Equal to the PKGNO of the response package |
| PAR2 | KEY Hex: 8bits bit0: front bit1: rear bit2: IR bit3: NUC | Response to a (KEY) package front:1=VCU Get “front” Key rear:1=VCU get “rear” key IR:1=VCU get “IR” key NUC:1=VCU get “NUC” key |
| PAR3 | String | Osd variant name |
| *CHK | checksum |  |

## 4.3 SYSCTL (from HOST to VDU)

| item | value | description |
| --- | --- | --- |
| $PKGID | $SYS |  |
| PAR1 | 0:disable 1:enable | System information output enable |
| *CHK | checksum |  |

## 4.4 SYSINFO (from VDU to HOST)

| item | value | description |
| --- | --- | --- |
| $PKGID | $SYSA |  |
| PAR1 | float | Power current (Unit: 0.001A) |
| PAR2 | float | CPU temperature (unit: 0.001C) |
| PAR3 | Integer: 0 - 100 | CPU usage percentage (unit: %) |
| PAR4 | Integer: 0 – 100 | Mem usage percentage (unit: %) |
| *CHK | checksum |  |

This data package will be send per 500ms when HOST enable the information output in the SYSCTRL package. It will continue to be send until the HOST disable it.

## 4.5 DEV Package(from HOST to VDU)

| item | value | description |
| --- | --- | --- |
| $PKGID | $DEV |  |
| PAR1 | Integer: 0 to 255 | Package No |
| *CHK | checksum |  |

## 4.6 Response of “DEV Package”(from VDU to HOST)

| item | value | description |
| --- | --- | --- |
| $PKGID | $DEVA |  |
| PAR1 | string | System Version |
| PAR2 | string | Serial No |
| PAR3 | string | Manufacturing date |
| *CHK | checksum |  |