Communication Protocol of Serial Port between B3VDU and B3VCU

Revision History

| Name | Date | Reason For Changes | Version |
| --- | --- | --- | --- |
| Chen Yong | Feb 22, 2022 | Initial draft | 1.0.1 |
| Chen Yong | Jul 14, 2022 | NUC mode | 1.0.2 |
| Chen Yong | Aug 18, 2022 | OSD Variant | 1.0.3 |

Table of Contents

1 Scope	4

2 Hardware Specification	4

3 system architecture	4

3.1 Data package	4

3.2 Communicaite mechanism	4

4 Data Format	5

4.1 Key package(from VDU to HOST)	5

4.2 Response of “Key Pacakge”(From Host to VDU)	5

4.3 SYSCTL(from HOST to VDU)	6

4.4 SYSINFO(from VDU to HOST)	6

4.5 DEV Package	6

4.6 Response of “DEV Package”	7

# Scope

This document stipulates communication protocol of serial port in-Between B3VDU and B3VCU. This document will be used as a guideline to serial port communication of B3 VDU and B3VCU according to the technical specification requirement.

# Hardware Specification

Physical link:TIA/EIA-422

baudrate:115200 bits/s

data bits:8 bits

stop bits:1 bit

parity: no

flow contorl:no

mode: duplex

# system architecture

VDU

HOST

(/VCU)

## Data package

| header | Package ID | parameters | tail | Checksum | End symbol |
| --- | --- | --- | --- | --- | --- |
| $ | Ascii code | Ascii code. Insert ‘,’ between parameters | * | 2 HEX ascii | \r or \n or \0 or (any combination) |

checksum: the two HEX character as the checksum of the package.(the sum of all chracters of package between ‘$’ and  ‘*’, exclude ‘$’ and ‘*’). It is not a neccesary item. It can be empty.

Example:

$KEY,123,0A,01234567*10\0

$KEY,123,0A,01234567*\r\n\0

## Communicaite mechanism

- VDU continuse to send “Key package” per 100 ms.

- VDU will set the HOST error bit if it does not receive the response package of “Key package” within 100ms.

- VDU can recieve the “sysctl” command pacakge.

- VDU will start or stop sending “sysinfo pacakge” per 500ms based on the “sysctl” command.

- VDU can recieve the “dev” command package.

- VDU will send the device information back in the Response of “dev Package”.

# Data Format

## Key package(from VDU to HOST)

| item | value | description |
| --- | --- | --- |
| $PKGID | $KEY |  |
| PAR1 | Integer: 0 to 255 | Package No |
| PAR2 | Hex: 8bits bit0: front bit1: rear bit2: IR bit3: NUC bit7: NUC mode (note: front and rear cannot be 1 at the same time in the package.) | Key (bit0-bit3) 0=released 1=pressed NUC mode: 0=auto 1=development |
| PAR3 | Hex: 32bits Bit0: memory bit1: logdata bit2: snsor bit3: current bit4: temperature bit5: pwm0 bit6: pwm1 bit7: keypad bit8: video out bit9: video in0 bit10: video in1 bit11: osd bit12: serial bit13: host | System error bits. 0: PASS 1: Fail |
| PAR4 | Integer | System Power On timer(unit:s) |
| *CHK | checksum |  |

## Response of “Key Pacakge”(From Host to VDU)

| item | value | description |
| --- | --- | --- |
| $PKGID | $KEYA | Acknowledge of package ‘KEY’ |
| PAR1 | Integer: 0 to 255 | Equal to the PKGNO of the response package |
| PAR2 | KEY Hex: 8bits bit0: front bit1: rear bit2: IR bit3: NUC bit7: IR Mode | Response to a (KEY) package front:1=VCU Get “front” Key rear:1=VCU get “rear” key IR:1=VCU get “IR” key NUC:1=VCU get “NUC” key IR Mode:1=IR ON 0=IR OFF |
| PAR3 | String | Osd variant name |
| *CHK | checksum |  |

## SYSCTL(from HOST to VDU)

| item | value | description |
| --- | --- | --- |
| $PKGID | $SYS |  |
| PAR1 | 0:disable 1:enable | System information output enable |
| *CHK | checksum |  |

## SYSINFO(from VDU to HOST)

| item | value | description |
| --- | --- | --- |
| $PKGID | $SYSA |  |
| PAR1 | float | Power current(unit: 0.001A) |
| PAR2 | float | Cpu temperature (unit: 0.001C) |
| PAR3 | Integer: 0 - 100 | Cpu usage percentage(unit:%) |
| PAR4 | Integer: 0 – 100 | Mem usage percentage(unit:%) |
| *CHK | checksum |  |

This data package will be send per 500ms when HOST enable the information output in the SYSCTRL pacakge. It will continue to be send until the HOST diable it.

## DEV Package

| item | value | description |
| --- | --- | --- |
| $PKGID | $DEV |  |
| PAR1 | Integer: 0 to 255 | Package No |
| *CHK | checksum |  |

## Response of “DEV Package”

| item | value | description |
| --- | --- | --- |
| $PKGID | $DEVA |  |
| PAR1 | string | System Version |
| PAR2 | string | Serial No |
| PAR3 | string | Manufacturing date |
| *CHK | checksum |  |